Home Office Network Blueprint v1.12

Status: Production – Secure OTAP Platform – HTTPS Enabled – Monitoring Active – Internal DNS Active – Secure VPN Access

Nieuwe toevoegingen sinds v1.11

- WireGuard VPN gateway (ER7206)

- VPN subnet segmentation

- Developer vs Tester VPN access model

- SSH access restriction (LAN + VPN only)

- Structured VPN IP allocation

- Secure remote developer access

Inhoudsopgave

1️⃣ Netwerkarchitectuur Overzicht

2️⃣ Naming Convention

3️⃣ Device Codes

4️⃣ Backbone Mapping

5️⃣WAN Status

6️⃣ DHCP Architectuur

7️⃣WiFi Architectuur

8️⃣ NAS Architectuur

9️⃣ NAS Security

Firewall Hardening

1️⃣1️⃣ Compute Layer — Rocky Linux Server

1️⃣2️⃣ Container Runtime

1️⃣3️⃣ OTAP Directory Architectuur

1️⃣4️⃣ Container Network Architectuur

1️⃣5️⃣ Reverse Proxy Layer

1️⃣6️⃣ HTTPS / TLS Architectuur

1️⃣7️⃣ SSH Security

1️⃣8️⃣ Monitoring Stack

1️⃣9️⃣ Internal DNS Platform

2️⃣0️⃣ VPN Architecture

1️⃣ Netwerkarchitectuur Overzicht

Live Architectuur

Internet
   ↓
Ziggo Modem (Bridge Mode)
   ↓
Firewall (TP-Link ER7206)
   ↓
Switch (TL-SG1016D)
   ↓
SRV / NAS / AP / DSK

Firewall fungeert als:

- Enige router

- Enige DHCP server

- Enige NAT

- Security perimeter

- Centrale logging node

- VPN gateway

2️⃣ Naming Convention

Formaat

[DEVICE]-[PORT] <-> [DEVICE]-[PORT]

Principes

- Geen richting afhankelijkheid

- 1-op-1 herleidbaar

- Topologie gedreven

- Consistente labeling

3️⃣ Device Codes

| Device | Code |
| --- | --- |
| Ziggo Modem | ZG |
| ER7206 Firewall | FW |
| TL-SG1016D Switch | SW |
| ThinkStation Server | SRV |
| QNAP NAS | NAS |
| Netgear WAX610 | AP |
| Desktop | DSK |

4️⃣ Backbone Mapping

| Van | Naar | Label |
| --- | --- | --- |
| ZG-LAN01 | FW-WAN02 | ZG-LAN01 <-> FW-WAN02 |
| FW-LAN03 | SW-01 | FW-LAN03 <-> SW-01 |

5️⃣ WAN Status

- Publiek WAN IP actief

- Geen 192.168.178.x netwerk

- Geen double NAT

- Ziggo modem werkt als bridge modem

Architectuur

Internet
   ↓
Firewall (Public IP)
   ↓
LAN 192.168.0.0/24

6️⃣ DHCP Architectuur

LAN subnet

192.168.0.0/24

Gateway

192.168.0.1

IP segmentatie

| Range | Gebruik |
| --- | --- |
| 192.168.0.1 | Firewall |
| 192.168.0.2 – 0.99 | Infrastructure |
| 192.168.0.100 – 0.200 | Dynamic clients |
| 192.168.0.201 – 0.254 | Future |

DHCP pool

192.168.0.100 – 192.168.0.200

Reservations

| Device | IP |
| --- | --- |
| SRV | 192.168.0.10 |
| NAS | 192.168.0.20 |
| AP | 192.168.0.30 |

DNS via DHCP

Primary DNS → 192.168.0.10 (AdGuard)

7️⃣ WiFi Architectuur

Access Point configuratie

- Access Point Mode

- DHCP disabled

- WPA3/WPA2 Personal

- 2.4GHz + 5GHz actief

- SSID identiek aan oude Ziggo WiFi

Resultaat

- WiFi volledig via AP

- Ziggo WiFi volledig uitgeschakeld

- Centrale netwerkcontrole via firewall

8️⃣ NAS Architectuur

Hardware

- 4 × 4TB HDD

- RAID 5

- Usable capacity: 10.89 TB

- Snapshot reserve: 10%

Volume indeling

| Volume | Grootte | Doel |
| --- | --- | --- |
| DEV | ~5TB | OTAP container data |
| DATA | ~3TB | Werkdata |
| MEDIA | ~1.4TB | Privé opslag |

Vrije poolruimte

≈ 200GB buffer

9️⃣ NAS Security

Accounts

- Default admin disabled

- qn-admin active

- 2FA enabled

Cloud exposure

- myQNAPcloud niet geconfigureerd

- Geen automatische port mapping

QuFirewall

LAN-ONLY profiel

Allow

192.168.0.0/24

Deny

All other networks

Firewall Hardening

Management

- HTTPS only

- HTTP redirect actief

- WAN management disabled

WAN exposure

- Geen port forwards

- Geen DMZ

- UPnP disabled

- WAN → LAN default deny

Attack defense

- SYN flood protection

- UDP flood protection

- ICMP flood protection

- TCP stealth scan block

- Packet anomaly protection

Logging

- System log enabled

- NTP synchronization

1️⃣1️⃣ Compute Layer — Rocky Linux Server

Server

Hostname: srv-otap-01
IP: 192.168.0.10
OS: Rocky Linux 9 Minimal

Hardware

- Lenovo ThinkStation S30

- Xeon E5-2660 v2

- 256GB RAM

- 1TB SSD (OS)

- 1TB HDD (/srv)

Architectuur principe

Firewall = Perimeter
Server   = Compute
NAS      = Storage

1️⃣2️⃣ Container Runtime

Container platform

Podman (rootless)

Voordelen

- daemonless container runtime

- Docker CLI compatible

- rootless security model

- betere isolation

Podman storage

/srv/podman
 ├ graphroot
 └ runroot

1️⃣3️⃣ OTAP Directory Architectuur

Root

/srv/otap

Structuur

/srv/otap
├ dev
├ test
├ acc
├ prod
├ proxy
├ monitoring
├ dns
├ management
└ shared

1️⃣4️⃣ Container Network Architectuur

Podman networks

otap-proxy-net
otap-dev-net
otap-test-net
otap-acc-net
otap-monitoring-net
otap-dns-net
otap-management-net

Architectuur

NGINX
 │
otap-proxy-net
 ├ dev
 ├ test
 └ acc

otap-monitoring-net
 ├ Prometheus
 ├ Grafana
 └ Node Exporter

otap-dns-net
 └ AdGuard

otap-management-net
 └ Portainer

1️⃣5️⃣ Reverse Proxy Layer

Reverse proxy

NGINX

Container

otap-proxy-nginx

Ingress services

dev.otap.lan
test.otap.lan
acc.otap.lan
grafana.otap.lan
portainer.otap.lan
adguard.otap.lan

1️⃣6️⃣ HTTPS / TLS Architectuur

Certificaat model

Interne Root CA

Wildcard TLS

*.otap.lan

TLS termination

NGINX reverse proxy

1️⃣7️⃣ SSH Security

SSH hardening

- Key-based authentication

- Password login disabled

- Root login disabled

SSH access restriction

AllowUsers sulay@192.168.0.* sulay@192.168.50.*

Resultaat

| Source | SSH Access |
| --- | --- |
| LAN | ✔ toegestaan |
| VPN | ✔ toegestaan |
| Internet | ✖ geblokkeerd |

1️⃣8️⃣ Monitoring Stack

Monitoring netwerk

otap-monitoring-net

| Component | Container |
| --- | --- |
| Prometheus | otap-prometheus |
| Grafana | otap-grafana |
| Node Exporter | otap-node-exporter |
| NGINX Exporter | otap-nginx-exporter |

Grafana

https://grafana.otap.lan

1️⃣9️⃣ Internal DNS Platform

DNS server

AdGuard Home

Container

otap-adguard

Wildcard DNS

*.otap.lan → 192.168.0.10

2️⃣0️⃣ VPN Architecture

VPN protocol

WireGuard

Firewall

ER7206 VPN Gateway

VPN subnet

192.168.50.0/24

IP allocation

| IP | Device |
| --- | --- |
| 192.168.50.1 | Firewall VPN |
| 192.168.50.3 | Developer laptop |
| 192.168.50.10 | Tester laptop |
| 192.168.50.11 | Future tester |
| 192.168.50.12 | Future tester |

Access model

| User | AllowedIPs |
| --- | --- |
| Developer | 192.168.0.0/24 |
| Tester | 192.168.0.10/32 |

Resultaat

- Developer → volledige LAN toegang

- Tester → alleen OTAP ingress server

Architectuur Resultaat

✔ Secure remote development access
✔ VPN segmented access control
✔ Wildcard DNS platform
✔ Monitoring stack actief
✔ Container management UI
✔ Secure ingress gateway

Huidige Status

| Component | Status |
| --- | --- |
| Hardware | ✅ |
| DHCP Segmentatie | ✅ |
| Bridge Mode | ✅ |
| Firewall Hardening | ✅ |
| NAS Security | ✅ |
| Rocky Linux Server | ✅ |
| Podman Runtime | ✅ |
| Container Networking | ✅ |
| Reverse Proxy | ✅ |
| HTTPS / TLS | ✅ |
| SSH Hardening | ✅ |
| Monitoring Stack | ✅ |
| Internal DNS | ✅ |
| Container Management | ✅ |
| VPN Access | ✅ |
| Production Containers | 🔜 |

Volgende Strategische Stap

1️⃣ Podman auto-updates (Quadlet)
2️⃣ Real DEV application stack
3️⃣ PostgreSQL / Redis platform services
4️⃣ Container backup strategy
5️⃣ CI/CD pipeline voor OTAP deployments